Hand Tools Risk Assessment Bristol Parks and Green Spaces Volunteer Programme

Prepared by: Bristol Parks

Date: July 2026

| Hazard | Possible Outcome | Who at risk? | Control measures | Risk | Person responsible |
| --- | --- | --- | --- | --- | --- |
| Inexperienced volunteers | Tool being used incorrectly | Vols and public | Volunteers leaders should be trained in the correct use and storage of hand tools. A tool talk must be given to all volunteers before work commences. Training should be received on the correct use of Personal Protective Equipment. Do not use tools you are not familiar with, and do not know how to use in a safer way. | Medium | Health and safety lead |
| Other volunteers working close by | Possible injury and possible obstruction. | Vols and public | A safe working distance must be maintained between volunteers. With swinging tools this is 2 times the length of the tool and the swinging arm. | M | Health and safety lead |
| Broken and damaged tools | Tool may break suddenly when in use and volunteer may lose control. | Vols and public | Inspect tools before use and do not use any that are damaged or have loose handles. Tools must be maintained in good condition and kept sharp. Tool users to use tools as instructed and inform lead if there is damage. | M | Health and safety lead |
| Swinging tools (for example, slasher, scythe, sledgehammer, mattock) | Tool may slip from grasp. | Vols and public | Only ‘gripper gloves’ / leather gloves may be worn when using swinging tools as others reduce grip. Swinging tools must not be used if handles are wet. Only use swinging tools if group lead has attended Health and Safety training, volunteers are familiar with the tool and how to use it, and there is a first aider present. | M | Health and safety lead |
| Sharp tools (for example, saws; slashers; axe, scythe, billhook) | Cuts and abrasions while handling and in use | Vols and public | Only use sharp, higher risk tools that you (lead volunteer) are familiar with and can demo safer use. Monitor volunteers' safe use. Move and store sharp-edged tools with guards on . Only use sharp, higher risk tools if there is a trained first aider in the work party, and a complete, up to date and accessible first aid kit. | M | Health and safety lead |
| Storage of tools on green space site while not in use | Tools may cause trip hazards, obstacles to walk into, be stolen and misused or lost. | Vols and public | Only get out tools that are in immediate use. Store unused tools in a clearly designated safe place and safe manner. | M | Health and safety lead |
| Transporting tools to and from work area | A tool may be carried incorrectly revealing a sharp edge or a hazard | Vols and public | Raise awareness of correct carrying methods and posture during tools talk. | M | Health and safety lead |