Activity Risk Assessment

Bristol Parks and Green Spaces Volunteer Programme

Activity: Scything

Prepared by: Bristol Parks and Green Spaces                  
Date: May 2025

If this risk assessment is relevant to your work session it must be identified on your site risk assessment. These hazards must be reassessed before an activity take place.

| Hazard | Possible Outcome | Who is at risk? | Control measures | Risk rating | Person responsible |
| --- | --- | --- | --- | --- | --- |
| Sharp blade and swinging action of tool | Injury from being cut by the blade | Group leader; public and scythe users / volunteers | Scythe users to maintain 2 scythes (Snath) distance apart in all directions. Bystanders to be kept a safe distance from scythe users. Use a cordon if necessary. Instructor / group lead to keep clear of Scythe user’s blades at all times. All scythe users must keep looking up, be aware of distances and warn others if they notice space or other issues. No one to stand behind a scythe user. | Medium | Person designated as health and safety lead on the H&S plan |
| Untrained scythe users | Incorrect / unpractised use of scythe leading to accident and injury of user or others | Scythe user and public | Supervision to maximum ratio of 8 scythe users or trainees per one trainer or trained leader Helpers to support trained leader for groups larger than 5. Leader of scything session has received Scything training (at least ½ day basic) Leader to check everyone is present when key instructions are given and to check understanding | Medium | Health and safety lead |
| Leading larger groups of scythe users or trainees (10 – 15 scythe users) | Harder for leader to monitor and supervise leading to incorrect use of scythe and potential injury | Scythe user and public | Group lead to recruit 1 trained helper / supervisor per 5 scythe users Designate areas or smaller groups for each lead and support to keep a closer eye on | Medium | Health and safety lead |
| Cleaning blade | Injury to person cleaning scythe | Scythe user | Users wear gloves for this activity. Users are shown correct way to clean blade. | Medium | Health and safety lead |
| Sharpening the blade – every 10 to 15 minutes during the mowing session | Injury to person sharpening | Scythe sharpener | Person sharpening the blade wears leather rigger style gloves for this activity and must know and use correct and safest way to sharpen the blade with sharpening stone and water. | Medium | Health and safety lead |
| Peening the blade – at least annually before mowing season begins | Injury to person peening the blade | Scythe sharpener | Use a peening jig which has been checked over and set up correctly on flat, stable ground. Person peening the blade must know and use correct method to set up and use the peening kit or not do this task. Person peening wears leather rigger style gloves. Use hammer with care. | Medium | Health and safety lead |
| Putting blade on/taking blade off snath | Injury to person putting blade on / taking it off | Scythe user | Users wear leather rigger style gloves for this activity. Users are shown correct way to put the blade on and take the blade off leather rigger style the snath. | Medium | Health and safety lead |
| Cutting with the scythe – uneven ground; hidden features or objects | Uneven ground leading to snags, slips and falls. Protruding objects leading to blade damage and jarring injury to scythe user. | Scythe user | Scythe users walk over the area to be scythed before starting work. Identify protruding objects (e.g. rocks or anthills etc) Inform other scythe users of features and obstacles to be aware of. | Low | Health and safety lead |
| Cutting with the scythe – plants with poisonous sap | Contact with poisonous plants and sap – for example, hogweed. Leading to blisters from contact with the skin | Scythe users / Volunteers working nearby | Volunteers to wear trousers and long-sleeved tops if working in an area where potentially hazardous plants are present | Low | Health and safety lead |
| Cutting with scythe | User losing control of scythe and slipping out of hands (especially in wet weather). Tool making contact with others leading to user or other injury. | Other people / dogs off leads in vicinity | Users do not wear gloves and work far away from others. Keep members of the public/people away from the work area. Keep an eye out for dogs not on leads. Do not scythe in wet weather when the handles (snaths) become wet and slippy. Ask group to warn each other of dogs / people approaching near. | Low | Health and safety lead |
| Carrying scythe | User slipping, tripping or losing control of scythe leading to injury | Scythe user/ other people in the vicinity | Users are shown the correct way to carry the scythe. Users are also asked to put the blade guard on where-ever possible. A check of the area being scythed is carried out prior to the course and any slip or trip hazards marked. | Medium | Health and safety lead |
| Storing scythes | Incorrect storage of scythe leading to injury through it falling and contact with person | Members of the public | Users are shown the correct way to store the scythe so that it won't fall on anyone. Users are also asked to put the blade guard on where-ever possible. Scythes are either stored where members of the public don't have access to them, or it is made obvious that they shouldn't touch them – i.e. with 'do not cross' tape. | Medium | Health and safety lead |